SECTION 05 54 00

METAL FLOOR PLATES

Note to Specifier:  This section covers steel floor plates fastened to support steel with the Hilti Powder Actuated X-FCP Floor Plate Fastening System.  Consult Hilti for assistance in editing this section for the specific application.

PART 1 – GENERAL

1.01	SUMMARY

- Section Includes:

  - Metal gratings fastened to support steel with powder-actuated grating fastening system.

- Related Sections:

- Section 05 12 00 - Structural Steel

1.02	REFERENCES

A.	ANSI A10.3 - Powder-Actuated Fastening Systems -- Safety Requirements.

B.	ASTM A153 - Standard Specification for Zinc Coating (Hot-Dip) on Iron and Steel Hardware.

C.	ASTM A786 - Standard Specification for Hot-Rolled Carbon, Low-Alloy, High-Strength Low-Alloy, and Alloy Steel Floor Plates.

D.	ASTM A793 - Standard Specification for Rolled Floor Plate, Stainless Steel.

E.	ISO 9001 - Quality Management Systems -- Requirements.

F.	ISO 14001 - Environmental Management Systems -- Specification with Guidance for Use.

1.03	SUBMITTALS

A.	Comply with Section 01330 - Submittal Procedures.

B.	Product Data:

1.	Submit floor plate manufacturer's product data, including load tables.

2.	Submit fastener manufacturer's product data, including installation instructions.

C.	Shop Drawings:  Submit floor plate manufacturer's shop drawings, indicating material, surface pattern, thickness, dimensions, tolerances, clearances, span, fabrication, shop welding, supports, cutouts, stair treads, finish, and location of fasteners.

D.	Samples:  Submit fastener manufacturer's samples of floor plate fasteners.

E.	Manufacturer's Certification:  Submit fastener manufacturer's certification indicating supplied fasteners comply with manufacturer’s published specifications.

1.04	QUALITY ASSURANCE

- Floor Plate Fastener Manufacturer:  ISO 9001 and ISO 14001 certified company.

- Floor Plate Fasteners:  Lloyd’s Registered and ABS Type Approved.

- Field Measurements: Take field measurements before preparation of shop drawings and fabrication, as necessary.

1.05	DELIVERY, STORAGE, AND HANDLING

- Delivery:

1.	Floor Plates:  Deliver floor plates to site with labels on each panel clearly identifying manufacturer, panel size, and location of installation.

2.	Fasteners:  Deliver floor plate fasteners to site in manufacturer's original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:

1.	Floor Plates:  Store floor plates flat, in clean, dry area, off ground in accordance with floor plate manufacturer's instructions.  Protect plate edges from damage.

2.	Fasteners:  Store floor plate fasteners in clean, dry area in accordance with fastener manufacturer's instructions.

C.	Handling:  Protect materials and finish from damage during handling and installation.

PART 2 – PRODUCTS

2.01	MANUFACTURERS

- Steel Floor Plate Manufacturers:

1.	____________________

2.	____________________

3.	____________________

- Floor Plate Fastener Manufacturer:

1.	Hilti, Inc., PO Box 21148, Tulsa, Oklahoma 74121.  Toll Free (800) 879-8000.  Fax (800) 879-7000.  Technical Support (800) 879-6000 x6337.  Web Site www.us.hilti.com.

2.	Other manufacturers will not be allowed.

2.02	STEEL FLOOR PLATES

- Floor Plates:

1.	Carbon Steel:  ASTM A786.

2.	Stainless Steel:  ASTM A793, Type [304]  [304L]  [316]  [316L].

3.	Raised Surface Pattern:  [Manufacturers standard pattern]  [Pattern [A]  [B]  [C]].

4.	Thickness:  [1/4 inch]  [5/16 inch ]  [3/8 inch]  [1/2 inch]  [ _____ inch]  [As indicated on the drawings].

2.03	FLOOR PLATE FASTENERS

Note to Specifier:  Two different types of Floor Plate Fasteners are specified in this section.  The Hilti Powder Actuated X-FCP Floor Plate Fastening System is specified in this section to secure floor plates to support steel with removable type fastenings.  The X-FCP Floor Plate Fastening System consists of two components: an 8 mm threaded stud (either X-ST-GR M8 or X-BT M8) and the X-FCP Fastening Disk.  The X-ST-GR M8 stud is SAE 316 equivalent for non-corrosive and corrosive environments.  The X-BT M8 is SAE316 equivalent for corrosive environment applications where the stud fastener point must not penetrate the support steel.  The X-BT M8 is available with (SN12-R) and without (-R) an SAE 316 stainless steel washer with bonded black elastomeric sealing washer.

- Fasteners:

1.     Hilti Powder Actuated X-FCP Floor Plate Fastening System.

a.   Fastening Disks:  Assembly of 52 mm diameter disk, elastomeric sealing ring, and 8 mm internally threaded screw.

- Carbon Steel:  SAE 1008.  Hot-dip zinc galvanized, ASTM A153 to a minimum of 65 micrometers thickness.

- Stainless Steel:  SAE 316 or equivalent.

b.   Threaded Studs:  8 mm powder-actuated threaded studs fastened to support steel in field with Hilti powder-actuated fastening tool equipped with floor plate adapter that fits through pre-drilled holes in floor plates and contacts support steel.

- Stainless Steel:  SAE 316 or equivalent.

PART 3 – EXECUTION

3.01	EXAMINATION

A.	Examine support steel to receive steel floor plates for correct size, layout, and alignment.  Notify Engineer if supports are not acceptable.  Do not begin installation until unacceptable conditions have been corrected.

3.02	INSTALLATION - STEEL FLOOR PLATES

A.	Install steel floor plates in accordance with floor plate manufacturer's instructions at locations indicated on the drawings.

B.	Ensure support steel bearing surfaces are clean.

C.	Install floor plates level, square, rigid, with flush installation, and without warp.

D.	Support floor plates on all sides and attach to support steel with a minimum of two fasteners per full-size panel width.

E.	Cut openings for penetrations accurately to size and location.

F.	Cut openings for pipes a minimum of 2 inches larger in diameter than pipe.

G.	Replace defective or damaged floor plates as directed by Engineer.

3.03	INSTALLATION - FLOOR PLATE FASTENERS

A.	Install floor plate fasteners in accordance with fastener manufacturer's instructions.

B.	Drill 3/4-inch nominal diameter holes through floor plates at locations of fasteners.

C.	Fasten threaded studs to support steel through pre-drilled holes in floor holes using fastening tools provided by fastener manufacturer.

D.	Operate powder-actuated fastening tools only with operators trained and qualified by fastener manufacturer's authorized instructor in accordance with ANSI A10.3.

Note to Specifier:  X-BT M8 threaded studs must be installed into pre-drilled holes (4 mm diameter by 3/16 inch deep) in the support steel. Contact Hilti for X-BT M8 installation instructions.

E.	Drive threaded studs into support steel to proper embedment depth in accordance with fastener manufacturer's instructions.

F.	Fasten threaded studs perpendicular to support steel.

G.	Tighten fastening disk with sealing ring onto threaded stud with maximum tightening torque depending on thickness of support steel in accordance with fastener manufacturer's instructions.  Use tightening tool provided by fastener manufacturer.

3.04	FIELD QUALITY CONTROL

A.	Fastener Manufacturer's Field Services:  Fastener manufacturer's authorized instructor shall train and qualify operators for manufacturer's powder-actuated fastening tools in accordance with ANSI A10.3.

3.05	PROTECTION

A.	Protect installed steel floor plates and fasteners from damage during construction.

END OF SECTION